SOP-10103 for Biological Spill

| Spillages in the laboratory |  |  |
| --- | --- | --- |
| 1 | Stand back and assess the situation. If infectious aerosols are a concern, evacuate the laboratory for 1 hr to allow aerosols to settle before attempting to clean up the spill. Cordon off the area to restrict access until the aerosols clear and the clean-up is complete. |  |
| 2 | Remove any contaminated clothing or personal protective equipment and place in a biohazard bag for disposal. Wash your hands! |  |
| 3 | Wear appropriate personal protective equipment (PPE), as a minimum this would include a lab coat, gloves and safety specs (consider the need for overshoes to protect shoes from contamination). ! If the material spilt is infectious by inhalation spill responders should wear respiratory protective equipment with p3 filtration* |
| 4 | Remove any solid/sharp objects using forceps or tongs and place in a rigid container e.g. sharps box. |  |
| 5 | Add an appropriate disinfectant* to the material spilt – for large quantities of liquid spilt (>50 ml) a concentrated disinfectant should be added to an effective final concentration, e.g. 5ml of neat disinfectant to a 100 ml spill would give a final concentration of 5%. (*see risk assessment for details of disinfectants effective against the agent in use). |  |
| 6 | Absorb liquid with paper towels and allow disinfectant to work for 10-30 minutes. See risk assessment for disinfectant contact times. |  |
| 7 | Clean up the spill - working in a circular motion from the outer rim of the spill to the centre. Do not use a dustpan and brush as this may generate airborne contamination. |  |
| 8 | Place the clean-up materials in a biohazard bag. |  |
| 9 | Clean down area with disinfectant followed by soapy water and dispose in biohazard bag. |  |
| 10 | Wash hands with soap and water. |  |
| 11 | Remove the PPE you are wearing and wash hands again - for large spills lab coats are to be autoclaved before being sent for cleaning. |  |
| 12 | Arrange for the correct disposal of the waste generated during the clean-up procedure. |  |
| 13 | Report the event to your line manager/ project supervisor and complete an online adverse event report. |  |

- Document Control

| Document Name | Standard Operating Procedure for Biological Spillages |
| --- | --- |
| Document Ref. Number | SOP-10103 |
| Revision | 2.0 |
| Date of Issue | July 2018 |
| Written By | Gretta Roberts |
| Amended by | Andy Lee |
| Reviewed by | Biological Hazards & Genetic Modification Sub-Committee |
| Contact Email | healthandsafety@swansea.ac.uk |

- Amendment Record

| Revision | Date | Amendment(s) |
| --- | --- | --- |
| 0 |  | Draft |
| 1 | July 2018 | New operating procedure |
| 2 | May 2021 | Reviewed |